# P3 Protocol Specification (MYLAPS Decoder)

# 1. Overview

Binary protocol with framed messages, CRC protection, and TLV-encoded payload.

# 2. Frame Structure

| Field | Size | Description |
| --- | --- | --- |
| START | 1 byte | 0x8E |
| VERSION | 1 byte | Protocol version |
| LENGTH | 2 bytes | Little endian |
| CRC | 2 bytes | CRC16 (0x1021) |
| FLAGS | 2 bytes | Usually 0 |
| TOR | 2 bytes | Type Of Record |
| BODY | variable | TLV encoded |
| END | 1 byte | 0x8F |

# 3. Escaping

| Original | Escaped |
| --- | --- |
| 0x8E | 0x8D 0xAE |
| 0x8D | 0x8D 0xAD |
| 0x8F | 0x8D 0xAF |

# 4. Record Types (TOR)

| TOR | Name |
| --- | --- |
| 0x0001 | PASSING |
| 0x0002 | STATUS |
| 0x0003 | VERSION_DECODER |
| 0x0004 | RESEND |
| 0x0015 | SESSION |
| 0x0024 | GET_TIME |
| 0x002D | SIGNALS |

# 5. TLV Encoding

| Field | Size |
| --- | --- |
| Type | 1 byte |
| Length | 1 byte |
| Value | N bytes |

# 6. STATUS Fields

| Type | Meaning | Notes |
| --- | --- | --- |
| 0x01 | Noise | 2 bytes |
| 0x06 | GPS | 1 byte (bool) |
| 0x07 | Temperature | Raw value |
| 0x0A | Satellites | 1 byte |
| 0x0C | Voltage | Value / 10 |
| 0x81 | Decoder ID | 4 bytes |
| 0x83 | Controller ID | 4 bytes |

# 7. Example: STATUS Message

[8E ... 02 00 ...]
Decoded:
- Noise: 0
- Temperature: 27
- Voltage: 10.9 V
- GPS: true
- Satellites: 3

# 8. Example: PASSING Message (generic)

Contains transponder passing data such as:
- Transponder ID
- Timestamp
- Loop ID
- Signal strength

Exact field layout depends on decoder configuration.

# 9. Example: SIGNALS Message (generic)

Used for signal diagnostics.
Typically includes raw loop/channel signal data.